SIMATS ENGINEERING

ASSESSMENT TOOL 1

NAME: DIVYA SHREE.K

REG.NO: 192521376

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

2. A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

SOLUTION:

1. Given

IP Address Block: 192.168.20.0/24

A /24 network contains

- Network Address = 192.168.20.0

- Total IP addresses = 256

The administrator divides the network into three subnets.

| Lab | Subnet Mask |
| --- | --- |
| Lab A | /26 |
| Lab B | /27 |
| Lab C | /28 |

Formula

Block Size = 256 − Last Octet of Subnet Mask

Lab A

Prefix

/26

Subnet Mask

255.255.255.192

Last octet = 192

Block Size

= 256 − 192

= 64

Therefore each subnet increases by 64

Subnets are

- 192.168.20.0

- 192.168.20.64

- 192.168.20.128

- 192.168.20.192

Since Lab A is assigned first,

Network Address = 192.168.20.0

Broadcast Address

Broadcast = Next Network − 1

= 192.168.20.64 − 1

= 192.168.20.63

Valid Host Range

First Host = Network + 1 = 192.168.20.1

Last Host = Broadcast – 1 = 192.168.20.62

Number of Hosts Formula

Host bits = 32 − Prefix

= 32 – 26 = 6

Total addresses = 2⁶

= 64

Usable Hosts = 64 − 2

= 62

Lab A Summary

| Item | Value |
| --- | --- |
| Network Address | 192.168.20.0 |
| First Host | 192.168.20.1 |
| Last Host | 192.168.20.62 |
| Broadcast Address | 192.168.20.63 |
| Usable Hosts | 62 |

Lab B

Lab A used addresses

0–63

So Lab B starts from 64

Prefix

/27

Subnet Mask

255.255.255.224

Last octet = 224

Block Size

256 − 224

= 32

Subnets increase by 32

- 64

- 96

- 128

- 160

- 192

- 224

Lab B starts at 64

Network Address

192.168.20.64

Broadcast Address

Next Network

192.168.20.96

Broadcast

96 – 1 = 192.168.20.95

Valid Host Range

First Host = 192.168.20.65

Last Host = 192.168.20.94

Number of Hosts

Host bits 32 – 27 = 5

Total addresses

2⁵ = 32

Usable Hosts 32 – 2 = 30

Lab B Summary

| Item | Value |
| --- | --- |
| Network Address | 192.168.20.64 |
| First Host | 192.168.20.65 |
| Last Host | 192.168.20.94 |
| Broadcast Address | 192.168.20.95 |
| Usable Hosts | 30 |

Lab C

Lab B used

64–95

Therefore Lab C starts from 96

Prefix

/28

Subnet Mask

255.255.255.240

Last octet

240

Block Size

256 – 240 = 16

Subnets increase by 16

96

112

128

144

...

Lab C starts at 96

Network Address 192.168.20.96

Broadcast Address

Next subnet

112

Broadcast

112 – 1 = 192.168.20.111

Valid Host Range

First Host = 192.168.20.97

Last Host = 192.168.20.110

Number of Hosts

Host bits 32 – 28 = 4

Total addresses 2⁴ = 16

Usable Hosts 16 – 2 = 14

Lab C Summary

| Item | Value |
| --- | --- |
| Network Address | 192.168.20.96 |
| First Host | 192.168.20.97 |
| Last Host | 192.168.20.110 |
| Broadcast Address | 192.168.20.111 |
| Usable Hosts | 14 |

Step 2: Final Answer Table

Subnet Mask; Network Address; First Host; Last Host; Broadcast Address; Usable Hosts. /26 (255.255.255.192); 192.168.20.0; 192.168.20.1; 192.168.20.62; 192.168.20.63; 62. /27 (255.255.255.224); 192.168.20.64; 192.168.20.65; 192.168.20.94; 192.168.20.95; 30. /28 (255.255.255.240); 192.168.20.96; 192.168.20.97; 192.168.20.110; 192.168.20.111; 14.

Question 2

Given

- System 1 IP Address: 192.168.1.130

- System 2 IP Address: 192.168.1.190

- Subnet Mask: 255.255.255.192 (/26)

Step 1: Find the Block Size

Formula

Block Size = 256 − Last Octet of Subnet Mask

Subnet Mask = 255.255.255.192

Last octet = 192

Block Size = 256 − 192 = 64

Therefore, the subnet ranges are:

| Subnet No. | Network Address | Broadcast Address | Host Range |
| --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.63 | 192.168.1.1 – 192.168.1.62 |
| 2 | 192.168.1.64 | 192.168.1.127 | 192.168.1.65 – 192.168.1.126 |
| 3 | 192.168.1.128 | 192.168.1.191 | 192.168.1.129 – 192.168.1.190 |
| 4 | 192.168.1.192 | 192.168.1.255 | 192.168.1.193 – 192.168.1.254 |

Step 2: Find the Subnet for System 1

Given IP

192.168.1.130

The IP lies between 128 and 191

Therefore,

Network Address

192.168.1.128

Broadcast Address

192.168.1.191

Valid Host Range

First Host

192.168.1.129

Last Host

192.168.1.190

Check Whether the IP Belongs to the Subnet

Given IP

192.168.1.130

Host Range

192.168.1.129 – 192.168.1.190

Therefore,

192.168.1.130 belongs to this subnet.

Step 3: Find the Subnet for System 2

Given IP

192.168.1.190

190 also lies between 128 and 191

Therefore,

Network Address

192.168.1.128

Broadcast Address

192.168.1.191

Valid Host Range

192.168.1.129 to 192.168.1.190

Check Whether IP Belongs to the Subnet

Given IP 192.168.1.190

Host Range 192.168.1.129 – 192.168.1.190

Therefore,

192.168.1.190 belongs to this subnet.

22

Step 4: Number of Usable Hosts

Prefix = /26

Host bits = 32 – 26 = 6

Total IP Addresses = 2⁶ = 64

Usable Hosts = 64 – 2 = 62

Question 3

Given

Network Block: 172.16.0.0/16

Departments:

- HR → /25

- Sales → /26

- IT → /27

- Admin → /28

Step 1: HR Department (/25)

Subnet Mask

255.255.255.128

Block Size

256 − 128 = 128

The first available subnet is:

Network Address

172.16.0.0

Broadcast Address

Next subnet = 172.16.0.128

Broadcast = 172.16.0.127

Valid Host Range

First Host

172.16.0.1

Last Host

172.16.0.126

Number of Usable Hosts

Host bits

32 − 25 = 7

Total addresses

2⁷ = 128

Usable Hosts

128 − 2 = 126

Step 2: Sales Department (/26)

After HR, the next available address is 172.16.0.128

Subnet Mask

255.255.255.192

Block Size

256 − 192 = 64

Network Address

172.16.0.128

Broadcast Address

Next subnet

172.16.0.192

Broadcast

172.16.0.191

Valid Host Range

172.16.0.129 to 172.16.0.190

Usable Hosts

Host bits

32 − 26 = 6

Total addresses

2⁶ = 64

Usable Hosts

64 − 2 = 62

Step 3: IT Department (/27)

Next available address

172.16.0.192

Subnet Mask

255.255.255.224

Block Size

256 − 224 = 32

Network Address

172.16.0.192

Broadcast Address

Next subnet

172.16.0.224

Broadcast

172.16.0.223

Valid Host Range

172.16.0.193

to

172.16.0.222

Usable Hosts

Host bits

32 − 27 = 5

Total addresses

2⁵ = 32

Usable Hosts

32 − 2 = 30

Step 4: Admin Department (/28)

Next available address

172.16.0.224

Subnet Mask

255.255.255.240

Block Size

256 − 240 = 16

Network Address

172.16.0.224

Broadcast Address

Next subnet

172.16.0.240

Broadcast

172.16.0.239

Valid Host Range

172.16.0.225

to

172.16.0.238

Usable Hosts

Host bits

32 − 28 = 4

Total addresses

2⁴ = 16

Usable Hosts

16 − 2 = 14

Step 5: Remaining Unused IP Address Range

The assigned subnets use addresses:

- HR: 172.16.0.0 – 172.16.0.127

- Sales: 172.16.0.128 – 172.16.0.191

- IT: 172.16.0.192 – 172.16.0.223

- Admin: 172.16.0.224 – 172.16.0.239

The next unused address starts at:

172.16.0.240

Since the original network is 172.16.0.0/16, it ends at 172.16.255.255.

Therefore, the remaining unused range is:

172.16.0.240 – 172.16.255.25

Question 4

Given

- Network Address: 172.16.10.0/24

- Subnet Mask: /27

- IP Address 1: 172.16.10.78

- IP Address 2: 172.16.10.95

Step 1: Find the Subnet Mask

Prefix = /27

Subnet Mask = 255.255.255.224

Step 2: Find the Block Size

Formula

Block Size = 256 − Last Octet of Subnet Mask

Last octet = 224

Block Size = 256 – 224 = 32

Therefore, the subnet increments are:

- 0

- 32

- 64

- 96

- 128

- 160

- 192

- 224

Step 3: Calculate Total Number of Subnets

Original Network = /24

New Network = /27

Borrowed Bits

= 27 − 24

= 3

Formula

Total Subnets = 2^(Borrowed Bits)

= 2³

= 8 Subnets

Step 4: Calculate Number of Usable Hosts

Host Bits

= 32 − 27

= 5

Total Addresses

= 2⁵

= 32

Usable Hosts

= 32 − 2

= 30 Hosts

Step 5: List the Subnets

| Subnet | Network Address | Broadcast Address | Host Range |
| --- | --- | --- | --- |
| 1 | 172.16.10.0 | 172.16.10.31 | 172.16.10.1 – 172.16.10.30 |
| 2 | 172.16.10.32 | 172.16.10.63 | 172.16.10.33 – 172.16.10.62 |
| 3 | 172.16.10.64 | 172.16.10.95 | 172.16.10.65 – 172.16.10.94 |
| 4 | 172.16.10.96 | 172.16.10.127 | 172.16.10.97 – 172.16.10.126 |
| 5 | 172.16.10.128 | 172.16.10.159 | 172.16.10.129 – 172.16.10.158 |
| 6 | 172.16.10.160 | 172.16.10.191 | 172.16.10.161 – 172.16.10.190 |
| 7 | 172.16.10.192 | 172.16.10.223 | 172.16.10.193 – 172.16.10.222 |
| 8 | 172.16.10.224 | 172.16.10.255 | 172.16.10.225 – 172.16.10.254 |

Step 6: Determine the Subnet for IP Address 172.16.10.78

Given IP

172.16.10.78

It lies between

64 and 95

Therefore,

Network Address

172.16.10.64

Broadcast Address

172.16.10.95

Valid Host Range

First Host

172.16.10.65

Last Host

172.16.10.94

Step 7: Check Whether 172.16.10.95 Belongs to the Same Subnet

Given IP

172.16.10.95

The subnet is

Network = 172.16.10.64

Broadcast = 172.16.10.95

Since 172.16.10.95 is the broadcast address, it is not a usable host address.

Therefore,

- It belongs to the same subnet.

- It cannot be assigned to a host because it is the broadcast address.

Final Answer Table

| Item | Answer |
| --- | --- |
| Network | 172.16.10.0/24 |
| Subnet Mask | 255.255.255.224 (/27) |
| Block Size | 32 |
| Total Subnets | 8 |
| Usable Hosts per Subnet | 30 |
| IP Address | 172.16.10.78 |
| Network Address | 172.16.10.64 |
| Broadcast Address | 172.16.10.95 |
| Valid Host Range | 172.16.10.65 – 172.16.10.94 |
| Does 172.16.10.95 belong to same subnet? | Yes |
| Is 172.16.10.95 usable? | No, it is the Broadcast Address |

Question 5

Given IPv6 Address

2001:0db8:0000:0000:0000:ff00:0042:8329

Prefix Length = /64

Step 1: Compress the IPv6 Address

IPv6 Compression Rules

- Remove leading zeros in each block.

- Replace consecutive blocks of 0000 with :: (only once).

Given Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

Remove leading zeros:

2001:db8:0:0:0:ff00:42:8329

Replace consecutive zeros:

2001:db8::ff00:42:8329

Compressed IPv6 Address

2001:db8::ff00:42:8329

Step 2: Expand the Compressed Address

Compressed Address:

2001:db8::ff00:42:8329

The symbol :: represents three groups of 0000, so expand it back:

2001:0db8:0000:0000:0000:ff00:0042:8329

This matches the original address.

Step 3: Determine the Network Prefix (/64)

A /64 prefix means:

- The first 64 bits (first four blocks) represent the network.

- The last 64 bits (last four blocks) represent the interface (host) identifier.

IPv6 Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix:

2001:0db8:0000:0000::/64

Compressed Form:

2001:db8::/64

Step 4: Calculate Total Number of Host Addresses

Prefix = /64

Host Bits

= 128 − 64

= 64 bits

Formula

Total Host Addresses = 2^(Host Bits)

= 2⁶⁴

= 18,446,744,073,709,551,616

(approximately 18.4 quintillion host addresses)